Scanner.exe:  “The Man Pages”

This is a simple program that allows you to read from and write to the Symbol scanner LS 1220.  It also is capable of calculating the actual speed of a moving object built to the model train scale HO along with its relative speed.

The Text Boxes

There are three text boxes.  Two are for output, and one is simply a listing for referance to previous scans.  

The text box on the far left is where you can input a programming code to set up the scanner.   This must be sent as a string of characters in hexadecimal format without spaces. 

Ex.  0253544948313630340306 to put the scanner in constant scan and report mode

The format and parameters are found in the LS 1220 Product Referance Guide.  (The last two digits, or the LRC, is a check number that is gotten by adding all the other two-digit numbers, except the 02, and is 0 plus the last digit you get from the adding.

The middle text box is to set the code length.  Before a scan, you must set this to the length of the barcode you are going to scan.  If this is not done, you may not get the entire scan, or you might not even receive the scan at all.

The bottom text box is where you enter the distance between barcodes that you are going to use to calculate the speed of a moving object.  This is defaulted to 12 inches.

The far right box is a numbered listing of everything you have scanned so far.  Also, when speed is calculated, the relative speed is listed in this box.

The Buttons

Send 


· Sends the code typed in the text box to the scanner.  If the code is not readable, a message box will pop up saying that it is not a hex number.

Calculate Speed

· Calculates the speed from the time between the last two scans and the distance in the text box.  There must be an even number of scans.

Constant Scan and Report Mode

· Puts scanner in constant scan and report mode

Host Triggering Mode

· Puts scanner in host triggering mode so the laser can be turned on by an external trigger.

On

· Turns laser on.

Off

· Turns laser off.

Defaults

· Sets the default parameters of the scanner.

Set 

· Sets the scanner to read barcodes of the length in the text box above.

Clear List

· Clears the list of all previous scans.

Clear Buffer

· Clears the scanner buffer.  Useful in the case of a mistake made on the length of the barcode.

